UNIT-5

Dependability issues:

When looking at the dependability of WSNs it is important to acknowledge that failures of the system need to be acceptable under certain conditions, as ultimately ensuring no failures of any devices within a WSN cannot be achieved. In these circumstances it is necessary to investigate how these failures aﬀect the network, and how they can be mitigated. Failures cannot be avoided as ultimately devices always fail once all batteries have been depleted, or whilst the batteries are being replaced, possibly leading to the failure of the system. For instance, applications such as carbon monoxide monitoring cannot function while the device is not powered. Ultimately, it is necessary to plan for failure, as in the extreme case external interference in the form of radio jammers could cause system failures that cannot be avoided by operational procedures. For these reasons, the following deﬁnition of dependability is used within this work.

“The ability  of a system to avoid service failures that are more frequent or more severe than is acceptable”.

Security Challenges in Wireless Sensor Networks

Due to continue growth of wireless sensor networks, the need for more effective security mechanisms is also increasing. The security concerns of the sensor network should be addressed from the beginning of designing of the system as sensor networks interact with sensitive data and usually operate in hostile unattended environments. A detailed understanding of the capabilities and limitations of each of the underlying technology is required for secure working of wireless sensor networks. Every node in the WSN must be designed to provide the set of primitives that are necessary to synthesize interconnected topology as it is deployed and meeting strict requirements of power consumption, size, and cost. Security for group communications applications that require packet delivery from one or more senders to multiple receivers is more critical and challenging goal.

The challenges of security in WSN are totally different from traditional network security due to inherent resource and computing constraints. Sensor nodes are often deployed in large accessible areas that present the added risk of physical attack. Sensor networks also poses new security problems as they interact closely with their physical environments and with people. Most of the early proposed network techniques in the past assumed that all nodes are cooperative and trustworthy. However, this is not the case for many sensor network applications today, that require a certain amount of trust in the application. This is required in order to maintain proper network functionality. Consequently, the existing security mechanisms are inadequate resulting in new research directions and new ideas for properly addressing sensor network security.

Any compromise on these requirements can cause a huge destruction in the network.

A. Data Integrity :- Wireless sensor networks are mostly used for security purposes therefore data integrity is very important in such networks. Data integrity ensures that data packets received at destination is exactly the same transferred by the sender and no one in the middle alters that packet. Wireless sensor networks mostly works on broadcasting therefore it is more vulnerable to such security attacks.

B. Confidentiality :-Confidentiality of the network means that data transfer between sender and receiver will be totally secure and no third person can access it (neither read nor write). In military operations sensed data is very important so it may be transferred securely to achieve confidentiality and secure key distribution .

C. Authentication:- Authentication of a sensor node ensures that he is a legitimate sensor and has the right to send data as well as the sent message by that node has the right contents. In asymmetric cryptographic communication digital signatures are used to check the authentication of any message or user while in symmetric key MAC,s are used for authentication purpose.

D. Self Organization:- Sensor nodes as well as sink nodes may have flexibility to organize themselves according to changing situation especially in mobile scenarios or in case of nodes failure. In many cases some sensor nodes are failed to activate themselves or their energy may consume faster. Then the neighbour nodes may arrange themselves to control the new situation. However due to inherit problems in wireless sensor networks there are still certain issues need to be resolve.

E. Data Freshness:- Data Freshness means the time when that packet was sent is recent or not. For security and avoidance of self destruction data freshness is very important in wireless sensor networks. Because an attacker can send an expire packet to waste the network resources and also cause self destruction.

F. Availability:- In order to ensure the availability of network resources. The sensor nodes may survive for more time if it save its energy or properly utilize it. When there is no activity in the network or the situation is normal as accordingly then sensor nodes may go in sleep mode to save their energy and utilize it in emergency scenario. In normal situation only few nodes are in active mode of operation. Whenever there is an attack the base station is responsible to activate all sensor nodes in sleeping mode.

G. Flexibility:- Wireless sensor networks play an important role in emergency scenarios and battle field. Therefore the external conditions as well as demands of the user changes rapidly. So according to the nature of mission or changing conditions the sensor nodes may have flexibility to adopt these changes.

H. Secure Localization:- To locate the accurate position of the sensor node. Accurate location of a sensor node is very important for data forwarding as well as trust management. There are 2 main types of localization Range based and range free based. Range based approach is normally used in wireless sensor networks.

I. Robustness and Survivability:-The sensor network should be robust against various security attacks, and if an attack succeeds, its impact should be minimized. The compromise of a single node should not break the security of the entire network.

Threat & Attack Models

THREAT MODELS

Threat modelling has emerged in recent years as an essential component of achieving cost-effective security. A threat model identifies assets and threats to them, and considers how such threats may be realized.

A variety of domain specific threat models have been developed: e-voting systems, high performance cluster (HPC) platforms, smart cards, software defined radio, insider threats , mobile phones , and secure data storage. Traditional threats, such as those arising from data remanence, have also been addressed . Let us first consider the state of threat modelling with respect to the networks of interest.

A. Sensor Networks:- Many applications are envisaged for sensor networks, in civilian and military domains. The number of applications will grow as sensor technology becomes cheaper and more sophisticated. The extent to which sensors can be protected (or can protect themselves) against compromise is limited. Sensors, once deployed in the wild, are vulnerable to capture and compromise. Some useful threat work has appeared in the literature. The relevant properties of sensor networks: tree-structured routing; data aggregation; tolerable failures; in-network filtering and computation; sensors as routers; and phased transmission periods. All these features give rise to threats and challenges. An attack model is based on adversary goals, categorized as eavesdropping, disruption and hijacking. They also identify attacks on specific protocols: TinyOS beaconing; directed diffusion; geographic routing; minimum cost forwarding; low-energy adaptive clustering hierarchy; rumor routing; and energy conserving topology maintenance (GAF, SPAN). The adoption encrypted communications affects greatly the applicability of attacks described. For deployment of governmental agency networks we might assume some degree of protection. Captured sensors may be deconstructed and examined or else compromised by less sophisticated methods such as RF pulsing to cause circuitry to be disabled or destroyed. The time needed to compromise a mote by sophisticated means may be too great. An interesting account of the current feasibility of physical compromise can be found in. There is a fair amount of work around on threat models for sensor networks, but it is distributed across various papers.

Attack Model/ Types of attacks classification

Attacks on Wireless Sensor Network can be categorized into various categories on the basis of certain criteria. In the first category attack can be:

1. Mote Class or Laptop Class. In mote class attacker can interact with only few sensor nodes where the entire sensor nodes must have similar capabilities, whereas in case of a Laptop Class attacker can interact with more powerful devices like PDA’s, Laptops etc. A Laptop Class adversary can produce a huge amount of damage than Mote Class. Mote class adversary can effect only within small geographical area, but on the other hand Laptop Class adversary could have an effect on the entire network and even could block the entire sensor network.

2. Insider or Outsider Attack. In case of Insider attack the attacker has access to that node which has all the secret keys and is capable of participating in all the communications. In Outsider attack attacker has no access to Wireless Sensor Network. It is done by the unauthorized node that eavesdrop the packets exchanged between the sensor nodes during their communication.

3. Network Layer Attack. which are: Attacks at Physical Layer, at Data Link Layer and at Network Layer. At Physical Layer attacker mainly exhaust the resources available by transmitting the radio signals on a Wireless Channel.  At Data Link Layer the attacker violate the predefined protocols of the Link Layer. This kind of attack also leads to Denial of Service attack. At Network Layer attacker threatens the sensor applications and services. In this Localization and Aggregation are used to prevent from this attack.

MAJOR ATTACKS IN WSN

As most of the routing protocols for WSN are very simple, so they are more vulnerable to attacks . Attacks on Network Layer protocol fall into one of the following categories:

1. Denial of Service (DoS): Denial of Service (DoS) attack is produced by malicious nodes or users. The main intention behind this attack is that it is an attempt to make network resources unavailable to its legitimate users. As the network is flooded with huge requests by an adversary so that legitimate user cannot access the services of the network. In wireless sensor network several types of DoS attacks might be performed at different layers. At Physical Layer, DoS attack can be known as Jamming. In this, adversaries continuously transmit radio signals

as well as high energy signals so that wireless medium could be blocked. Jamming is further of two types: Constant Jamming and Intermittent Jamming. In case of Constant Jamming, there is a complete jamming of the entire network, whereas in case of intermittent jamming, sensor nodes are capable of communicating data periodically but not consistently.

1. Selective Forwarding: It is also a network Layer attack. In this, an adversary usually forwards some of the packets and drops rest of the packets containing vital information. This degrades the quality of service in WSN. If somehow attacker discards all the packets, then the receiver node becomes conscious that there must be some obstacle in between, so neighboring nodes decide to take another route. But to overcome this doubt adversary forwards a selective packets to the node rather than dropping all the packets.

2. Blackhole/Sinkhole Attack: In this attack the major intention of the malicious node (Blackhole ) is to attract the maximum traffic towards itself. An adversary makes assures to all the sensor nodes that it is also a compromised node and it will provide them the best quality route and even the shortest path to the base station. Then all the neighboring nodes of the adversary will start transmitting the packets to the adversary and when the whole of the traffic reaches the adversary, it can do anything with that information. Even it can perform selective forwarding attack i.e., to drop the crucial data and forwards the rest of the irrelevant packets to the base station.

3. Sensor networks are much more prone to sinkhole attacks as they have the common destination and a compromised node needs only to assure that it will provide high quality routes to the base station just to attract the maximum traffic towards itself.

4. Hello Attack: In this attack, malicious node having high radio transmission range broadcasts HELLO message to the neighboring sensor nodes to make them assure that it is also a legitimate node as well as it will provide shortest route to the base station. As a result, while sending the packets to the base station sensors nodes packets pass through the malicious node because it has made them an illusion that it is their legitimate neighboring node and hence get all the relevant data and attack the sensor network.

5. Sybil Attack: This type of attack mainly occurs in peer to peer network and detection of it is very difficult. We define Sybil attack as a malicious node which forges the false identity of many legitimate nodes. Whenever there is a communication between the two legitimate nodes adversary node occurs in between the interaction and proves the sender node that it is the one that the sender wants to exchange the data with by using the identity of receiver node. Adversary node takes all the information and hence can use selective forwarding, degrading of the data, etc.

6. Wormhole Attack: Wormhole is a critical attack in which attacker connects two distant points in the network using a low latency communication link called wormhole link. Once the link is established the adversary records the packet at one location in the network and replays them at the other end. This type of attack is a significant threat to the sensor network as it could even be performed at the initial phase when sensors discover their neighboring information.

7.  Traffic Analysis Attacks:- Often, for an attacker to effectively render the network in useless state, the attacker can simply disable the base station. To make matters worse, Authors  demonstrate,  two attacks that can identify the base station in a network without even understanding the contents of the packets. A rate monitoring attack posits that nodes close to the base station tend to forward more packets than those farther away from the base station. While in a time correlation attack, an attacker generates events and monitors to whom a node sends its packets.

8. Node Replication Attacks:-By copying the node ID of an existing node an attacker can add a node to an existing sensor network. A replicated node can severely disrupt a sensor network's performance; packets can be corrupted or even misrouted.  This can result in a disconnected network and false sensor readings

9. Physical Attacks:-Indeed, in hostile outdoor environments, the small form factor of the nodes, coupled with the unattended and distributed nature of their deployment makes them vulnerable to physical attacks. Physical attacks ruin sensors permanently, so the losses are irreversible. For instance, attackers can access cryptographic secrets, tamper with the associated circuitry, spoofing / modifying programming in the nodes, and/or replace them with malicious nodes all of these within the control of the attacker.

Quality of Service(QoS)

Wireless sensor networks (WSNs) are required to provide different levels of Quality of Services (QoS) based on the type of applications. Providing QoS support in wireless sensor networks is an emerging area of research. Due to resource constraints like processing power, memory, bandwidth and power sources in sensor networks, QoS support in WSNs is a challenging task.

In general, QoS refers to quality as perceived by the user or application. In networking community, QoS is interpreted as a measure of service  quality that the network offers to the end user or application. Figure below shows a general QoS model for network  which is redrawn from.  QoS has been defined as a set of service requirements to be fulfilled when transmitting a stream of packets from source to destination. In traditional data network, QoS defines certain parameters such as packet loss, delay, jitter, bandwidth etc. However, the QoS requirements in WSNs such as data accuracy, aggregation delay, coverage, fault tolerance and network lifetime etc. are application specific and they are different from the traditional end-to-end QoS requirements due to the difference in application domains and network properties. Although, some QoS solutions (like IntServ, DiffServ etc) are developed for traditional networks, these cannot be easily ported in WSNs due to

1) severe resource constraints in sensors nodes,

2) large scale and random deployment of sensors nodes and

3) application specific and data-centric communication protocols in WSNs.

Researchers have been working continuously towards QoS support in WSNs and have proposed some methodologies for that purpose. To name a few, Network Layer based QoS support in terms of routing protocols, Cross Layer based QoS support  and Middleware layer based QoS support are the most prominent types of approaches for QoS support in WSNs. It is envisioned that WSNs will gradually become pervasive in our daily life and will finally revolutionize the way we understand and manage our physical world. This trend drives the WSN to provide QoS support to meet service requirement of its diverse applications. This motivates us to explore this challenging area and bring to the focus the possible research problems and their solutions The rest of the paper is organized as follows. In Section 2 we have discussed the characteristics of WSN which pose challenge for QoS support. Section 3 makes a survey of the existing approaches for QoS support in WSN.

QoS  METRICS (PARAMETERS)

In this section we present the metrics that quantify the QoS requirements. These include delay, jitter, packet loss, throughput, energy consumption, bandwidth utilization, network lifetime, and cost .

Delay-

it indicates the length of time taken for a packet to travel from source to destination. It represents average data delay an application experiences when transmitting data. This parameter is intrinsic to communication, since end points are distant and information will consume some time to reach the other side. Delay is also referred to as latency. Delay time can be increased if the packets face long queues in the network (congestion) or crosses a direct route to avoid congestion. The delay can be measured either one-way (total time from source that sends a packet to the destination that receive it) or round trip (one way latency from source to destination plus one way latency from destination to source). The round trip delay is relatively   accurate way of measuring delay, because it excludes the amount of time that a destination system spends processing the packet. It only sends a response back when it receives a packet. Delay has to be minimum.

Jitter-it is the delay variation and is introduced by the variable transmission of the delay of the packets over the network. This can occur because of router’s internal queues behavior in certain circumstances (eg. Flow congestion), routing changes etc.  this parameter can seriously affect the quality of streaming audio and/or video. To handle jitter, it is needed to collect packets and hold them long enough until the slowest packet arrives in time, rearranging them to be played in correct sequence.

Packet loss- happens when one or more packets of the data being transported across the internet or a computer network fail to reach their destination. This loss of packets can be caused due to signal degradation, high loads on network links, packets that are corrupted being discarded or defect in network elements. Wireless Networks have higher probability of loss that is introduced by the air interface (eg. Interference caused by other systems, multiple obstacles (buildings, environment) in the path, multiple path fading etc.). Packet loss can be tolerant but only to a certain extent.

Throughput- It is the number of bits passed through a network in one second. It is the measurement of how fast the data can pass through an entity such as point or a network. So it should be maximum.

Energy consumption- This is the amount of energy consumed by the devices during the period s of transmitting, receiving, idle and sleep. Hence battery life is key parameter and hence energy consumption should be less.

Bandwidth- certain bandwidth requirements must be fulfilled. It is what the reporting of events require. Hence efficient bandwidth utilization is must.

Network lifetime-It is defined as the minimum time at which maximum number of sensors nodes are dead or shut down during a long run of simulation. So it should be long.

Cost- sometimes the sensors are deployed in large numbers to increase the efficiency of system. It is very costly to check quality of service of individual sensors used in a typical application domain Of WSN. Hence it is necessary to consider network as a whole or at least to use network in part. So cost should be low.

QoS  PROVISIONING

WSNs have two main approaches for QoS provisioning:

- Classic layered approach

- Cross-layer approach.

Layered approach achieves QoS provisioning with protocols that operate only in one individual layer of the WSN communication protocol stack, while cross-layer approach provides the desired QoS through the simultaneous interaction of multiple layers in WSN. QoS provisioning goal for all layers of the communication protocol stack is equal: Providing the desired QoS level to applications and users while decreasing energy consumption, and consequently prolonging network lifetime. Efficiency of the QoS provisioning mechanism in individual layers depends on layer capabilities. QoS metrics usually include: bandwidth, throughput, latency, delay, jitter, packet loss and coverage. However lately energy consumption is becoming standard part of QoS metrics.

QoS provisioning in the MAC, network and transport layer is given below. Since the MAC layer is the bottom layer, it has a huge impact on QoS provisioning in the upper layers. Therefore, it must guarantee one or more performance metrics to upper layers. Protocols in each layer are isolated and do not alter the protocols in upper or lower layers.

MAC Layer

Since the MAC layer is the bottom software layer, performance of all upper layers are depending on the MAC layer components and performance. QoS support in the network or transport layers cannot be provided without the assumption of a MAC protocol which solves the problems of medium sharing and supports reliable communication . Therefore, QoS provisioning in MAC layer is of significant importance for overall QoS in WSN.  QoS metrics that can be fulfilled at the MAC layer are the following :

• Minimizing medium access delay.

• Minimizing collisions.

• Maximizing reliability.

• Minimizing energy consumption.

• Minimizing interference and maximizing concurrency (parallel transmissions).

• Maximizing adaptivity to changes.

QoS mechanisms that can be used at the MAC layer are the following :

• Adaptation and learning: Adapts operation parameters of the sensor nodes to the current network condition in order to accommodate traffic load in a more efficient way. Over time, learning algorithms could predict network changing and take necessary adaptive operations.

• Error control: Reduces consumption, whilst providing reliable and fast delivery of the sensor data. It can be implemented in each layer of the communication protocol stack.

• Data suppression and aggregation: Reduces traffic load of the network and consequently minimizes radio communication, preserves energy and increases bandwidth. It can also be implemented in each layer of the communication protocol stack.

• Power control: Adjusts transmission power of the sensor nodes to the minimum power required for successful transmission. It is implemented in MAC layer as a joint physical-MAC layer solution.

• Clustering: Simplifies the synchronization and coordination by grouping set of neighboring sensor nodes. Consequently, it lowers energy consumption and improves reliability.

• Service differentiation: Prioritizes network traffic based on several criteria, and then forms several traffic classes. Different QoS levels are provided to different traffic classes.

Network Layer

Routing protocols are implemented primarily in the network layer and they are in charge of discovering and maintaining the routes in the network. Most of the routing protocols designed for WSNs consider energy efficiency as the main objective, with the assumption that data does not have stringent QoS requirements. They can be classified as data-centric, hierarchical, location-based or network-flow protocols.  In data-centric protocols, queries are posed for specific data and routing is performed using the knowledge that is aggregate or meta-data. Hierarchical protocols minimize energy consumption by dividing nodes into clusters. Location-based protocols take advantage of the location information to make routing techniques more efficient. In network flow protocols, route step is modeled and solved as a network-flow problem.

QoS metrics that can be fulfilled at the network layer are the following:

• Minimizing path latency/delay.

• Maximizing routing reliability.

• Minimizing energy consumption.

• Minimizing congestion probability.

• Providing effective sample rate.

Routing QoS techniques are:

• Multipath routing: Ensures delivery of QoS and improves reliability in data delivery by sending multiple copies of each packet through multiple paths from source to sink.

• Minimum cost forwarding: Finds the minimum cost path in a large WSN.

Transport Layer

Transport protocols run over the network layer protocols. They are designed to control source transmissions, and to provide reliable message delivery and congestion control.  QoS metrics that can be fulfilled at the transport layer are the following:

• Maximizing end-to-end reliability.

• Bandwidth/throughput fairness.

• Minimizing congestion probability.

• Minimizing energy consumption.

QoS mechanisms that can be used at the transport layer are the following:

• Loss recovery: Consists of two phases: (i) Loss detection and notification (continuity of sequence number is used to detect packet loss); and (ii) Retransmission recovery (can be performed end-to-end or hop-by-hop).

• Congestion control: Uses following mechanism to control congestion: congestion detection, congestion notification, and congestion mitigation and avoidance .

• Flow control: Ensures that the overall transmission rate of the sender never exceeds the reception rate of the receiver. There are two types of flow control mechanisms: closed-loop flow control and open-loop flow control .

• Source prioritization: Prioritizes sensed information based on their information content priority.

CLOCK SYNCHRONIZATION

For a number of protocols time synchronisation is required, this may be either global time, or a relative point in time common to a subset of devices. Global time synchronisation requires that some deﬁnitive time source is deﬁned as the reference start time. These are commonly measured either from the start time of a speciﬁc node, or from some external time source such as a networked sink node. The issue with WSN s is that there can be large latencies with communication, making time synchronisation diﬃcult. Another issue is the clock drift between devices due to the small manufacturing diﬀerences between crystals on each device, in the extreme case leading to time diﬀerences of 54 ms every hours . Other external factors such as power and temperature can also aﬀect the speed of the clock, however by much smaller margins than the original skew. These diﬀerences can be compensated for by using clock skew compensation , measuring the relative diﬀerences and then compensating accordingly. This method however only prolongs the time to become desynchronised, with periodic re-synchronisation ultimately being required . Depending on the complexity of the synchronisation algorithm, times accurate down to the microsecond can be ensured between devices.

SUPPORTING FAULT TOLERANT OPERATIONS

Fault tolerance technique prevents lower level errors from propagating into system failures. By the use of various types of structural and informational redundancy, such techniques either form a fault or detect a fault and then effects a recovery process which, if successful, prevents a system failure. If not, then there is a permanent internal fault, and the recovery process then usually includes some form of structural reconfiguration that prevents the fault from causing further errors. The mixture of fault tolerance techniques that complement the techniques used for fault prevention will be incorporated by fault tolerance system designs. Fault tolerance techniques are LEACH, HEED, DFCA and etc. Due to lack of power, physical damage, or environmental interference, some sensor nodes may fail or can be blocked in WSNs. The overall task of the sensor network should not be affected by the failure of sensor nodes.

“Fault tolerance is the ability of a system to continue providing its specified service despite of its component failures”.

It is carried out through fault detection and fault recovery process. Since the sensor nodes are prone to failure, WSNs must offer features such as: availability, reliability and fault tolerance ability, security etc.

Source of Faults in Real WSN Applications

Wireless sensor networks are commonly spread wide in harsh environment and are subject to faults in several layers of the system. A fault that causes in each layer of the system has the possibility to propagate to above levels. For instance, the entire node will be failed if any power failure of a node occurs. If this node relies on a routing path, then the messages of other nodes that are relying on the same routing path will not be delivered making an entire region of the network silent till the routing path is restored.

we will concentrate on faults that can occur in the sensor nodes up to the sink.

1) Node Faults: Nodes have various hardware and software components which can produce malfunctions. The sensor nodes were exposed to direct contact with water causing short circuits due to stress from the environment and inadequate enclosures. The observer reported that the large-scale deployment in a potatoes field indicated that the antennas from the nodes were quite brittle and would become loose while inserting the node into the packaging

2) Link Faults: Communication links between nodes are highly volatile in WSNs. There are constant changes in the routing paths due to instability of the links between nodes. Link between nodes will become faulty, when radio interference occurs. For Example, in agricultural fields the deployment of the nodes must be carefully planned to take into consideration that when plants start growing the link range between nodes is considerably reduced.

3) Sink Faults: On a higher level of the network, a device (sink) that collects all the data generated in the network and propagates it to the back-end system causes faults of its components. The sink can be deployed in areas where no permanent power supply is present, in such applications batteries together with solar cells are commonly applied to provide the amount of energy necessary. However, this traditional technique has proven to be inefficient. Although this worked perfectly for other experiments, in the glacial environment the sink suffered a power failure due to snow covering the solar cells for several days.

Taxonomy of Fault Tolerant Techniques

Recent research has developed several techniques that deal with different types of faults at different layers of the network stack. To assist in understanding the assumptions, focus, and intuitions behind the design and development of these techniques, the taxonomy of different fault tolerant techniques used in traditional distributed systems was given as:

a) Fault prevention: this is to avoid or prevent faults;

b) Fault detection: this is to use different metrics to collect symptoms of possible faults;

c) Fault isolation: this is to correlate different types of fault indications (alarms) received from the network, and propose various fault hypotheses;

d) Fault identification: this is to test each of the proposed hypotheses in order to precisely localize and identify faults;

FAULT DETECTION TECHNIQUES

The goal of fault detection is to verify that the services being provided are functioning properly, and in some cases to predict if they will continue to function properly in the near future. The simplest way to perform such a task is through visual observation and manual removal of incorrect values

- Self-Diagnosis

In many cases, nodes can identify possible failures by performing self-diagnosis. An approach where a node would perform a self-diagnosis based on the measurements of accelerometers to determine if the node suffers from an impact that could lead to hardware malfunctions. Using a similar approach, nodes could detect when they are being moved to a different location. Faults caused by battery exhaustion can be predicted when the hardware allows the measurement of the current battery voltage. By analyzing the battery discharge curve and the current discharge rate, an algorithm can determine an estimation of the time to death of the battery.

B. Group Detection

The detection of services failing due to incorrectly generated values is only possible if a reference value is available. Detection mechanisms are proposed to identify faulty sensor node. Algorithms are based on the idea that sensors from the same region should have similar values unless a node is at the boundary of the event-region. The algorithms  start by taking measurements of all neighbors of a node and uses the results to calculate the probability of the node being faulty.

C. Hierarchical Detection

The definition of a detection tree enables a scalable fault detection algorithm in WSN. Memento  proposes the usage of the network topology to forward the fault detection results of child nodes to the parent nodes and up to the sink. Each node forwards the status of the child nodes that it is monitoring to its parent node. The parent performs an aggregation (bitwise OR) operation on the results of the child nodes together with its on results and forwards it to the next level.

FAULT RECOVERY TECHNIQUES

Fault recovery techniques enable systems to continue operating according to their specifications even if faults of a certain type are present. There are many potential sources for faults in WSNs. Fault tolerance techniques have been proposed in various contexts that increase the reliability of the functionality of sensor nodes in their specific domain.

A. Active replication in WSN:-Active replication in wireless sensor networks is naturally applied in scenarios where all, or many, nodes provide the same functionality. One example is a service that periodically provides sensor data. Nodes that run this service activate their sensors and forward their readings to an aggregation service or to a base station. When some nodes fail to provide that information, the recipient still gets the results from other nodes, which is often sufficient. Fault recovery in the presence of active replicas is relatively straightforward. Some of these approaches here:

1) Multipath routing: Usually, it is desirable to avoid that a single failing node causes the partitioning of a sensor network. Thus, a network should be k-connected, which allows k − 1 nodes to fail while the network would still be connected . Multipath routing  can be used to actively replicate routing paths

2) Sensor value aggregation: Sensor value fusion is a reasearch area that seeks to provides highlevel information derived from a number of low-level sensor inputs. There, the inherent redundancy of sensor nodes can be used to provide fault-tolerant data aggregation.This is achieved through a tradeoff between the precision (the length) of the resulting sensor reading interval and the number of faulty sensors. This ensures that despite of node failures, the resulting reading interval will contain the correct sensor reading of a region.

3) Ignore values from faulty nodes: A simple but efficient solution to not propagate a failure of one specific node to the entire network is to ignore the data that it is generating, as applied in . The major challenge in this case is the identification of the malfunctioning nodes.

B.Passive replication

in WSN When passive replication is applied, the primary replica receives all requests and processes them. In order to maintain consistency between replicas, the state of the primary replica and the request information are transferred to the backup replicas. The process of recovering from a fault when using passive replication is illustrated in Figure 4 and consists of three main steps: fault detection.

1) Node selection: After it has been established that a certain functionality is not available any longer due to a failure in the primary replica, a new service provider must be selected. After this selection phase, one or several nodes become service providers. Several approaches to how the selection is performed have been proposed. We differentiate them according to who makes the decision on which party should become a service provider

a) Self election: In LEACH , nodes periodically execute a probabilistic algorithm to establish whether they should serve as clusterhead to their neighbors. In this probabilistic rotation system, nodes keep changing their role in the network. When a clusterhead node fails, it will take only one rotation period until another node starts providing the functionality of the failed or absent node. Role assignment algorithms determine which of a certain role, such as coverage, clustering, and in-network aggregation, should be assumed by a node. In , a deterministic algorithm for autonomous role assignment is proposed that takes into account properties of the node such as battery status and location, but also its neighborhood and the roles chosen by neighboring nodes. This facilitates the localized self-configuration of a sensor network and can reestablish service provisioning if executed after some nodes have failed.

b) Group election: In , a reallocation of nodes that were part of a cluster that suffered a clusterhead failure is proposed. The clusterhead, called gateway, is considered to be a resourceful node. The solution presented considers that all the gateways in the network maintain a list of the nodes that are currently in their cluster and another backup list of nodes that could become part of their cluster. When a gateway fails, the nodes from its cluster are reallocated to the other gateways that have the nodes in their backup lists. If more than one gateway has a specific node in its backup list the node is assigned to the clusterhead that has the smallest communication cost.

c) Hierarchical election: In a hierarchical election, a coordinator selects the new primary node. This applies to the rebuilding of routing paths  as well as the selection of a new clusterhead. The former describes an algorithm to select the node that is closest to the base station. The latter approach applies fuzzy logic in the base station to select which node will become a clusterhead.

2) Service Distribution: During this phase, nodes elected to become service providers must activate the service. In some cases the service is already available on the nodes and a simple configuration change to inform the node that this service should be activated is required. However in some cases, for instance when nodes do not have enough memory to store the code of all potential services, it is necessary to inject code into the node through some technique. There are different techniques that can be used for service distribution: completely reprogramming the node, sending entire blocks of executable code, or sending small pieces of code such as scripts.

a) Pre-Copy: Pre-copying as described in , consists in making the code of all services available on all nodes before deployment. This allows nodes to change their behavior according to the role that they are assigned to.

b) Code distribution: Several approaches have been proposed for disseminating code throughout the network. Mate is an example for a bytecode interpreter for ´ TinyOS where code is broken into capsules of 24 instructions. These capsules can be distributed through the network and installed on nodes, which start to execute the new code. Agilla is a Mate-based mobile agent mid- ´ dleware for programming wireless sensor networks. These mobile agents can be programmed to move through the network or replicate themselves to other nodes according to changes in the environment. Impala  is a middleware for sensor networks that supports software updates and on-the-fly application adaptation. Unlike Mate, the focus of Impala is networks that ´ have a high degree of mobility, which can lead to long delays until an update is finished. While Mate stops the ´ execution of an application until the update is finished, Impala processes ongoing software updates in parallel.

c) Remote Execution: On the one hand code migration is an approach that reduces the amount of memory required in the entire network since not all nodes need to have the application pre-installed, on the other hand it consumes energy on the nodes exchanging the code and is susceptible to link failures, which could cause long delays until the code update is completed. Remote Execution  is an alternative approach where low power devices transfer tasks to more powerful devices without transferring the entire application code. Instead, only the required state information is transmitted. Such an approach is especially suited for heterogeneous sensor networks with at least some resourceful nodes. A hybrid approach between code migration and remote execution is proposed in , where the application code is copied to another node when the battery level reaches a first threshold. As soon as the battery reaches a critical level, the execution state is transferred and control is handed to the remote node. This allows for the full usage of the available energy resources, since control is handed over right before a node fails.

INTRODUCTION TO OPERATING SYSTEM

WSNs can be used to monitor and/or control physical environment in a space where it is difﬁcult or impossible to do so manually. A WSN is generally composed of a centralized station (sink) and tens, hundreds, or perhaps thousands of tiny sensor nodes such as Mote  and Mica2 . The information is often unidirectional ﬂow from the sensor nodes to the sink. Since a WSN has a centralized sink and unidirectional information ﬂow, it acts as a centralized system. But the sensor nodes are distributed and behave collaboratively. At the same time, a WSN is not only a database system but also a resource-constrained network with most networking functions, so they are often used to monitor events and collect data. Therefore, the environment is event driven and data centric. Therefore, WSNs are a special type of distributed network system that is similar to database, real-time, and embedded systems.

The traditional OS is system software that operates between application software and hardware and is often designed for workstations and PCs with plenty of resources. This is usually not the case with sensor nodes in WSNs. Sensors usually have a slow processor and small memory, different from most current systems.

Traditional OSs are not suitable for wireless sensor networks because WSNs have constrained resources and diverse data-centric applications, in addition to a variable topology. WSNs need a new type of operating system, considering their special characteristics. There are several issues to consider when designing operating systems for wireless sensor networks.

1. TinyOS

The design of Tiny OS allows application software to access hardware directly when required. TinyOS is a tiny microthreaded OS that attempts to address two issues: how to guarantee concurrent data ﬂows among hardware devices, and how to provide modularized components with little processing and storage overhead. These issues are important since TinyOS is required to manage hardware capabilities and resources effectively while supporting concurrent operation in an efﬁcient manner. TinyOS uses an event-based model to support high levels of concurrent application in a very small amount of memory. Compared with a stack-based threaded approach, which would require that stack space be reserved for each execution context, and because the switching rate of execution context is slower than in an event-based approach, TinyOS achieves higher throughput. It can rapidly create tasks associated with an event, with no blocking or polling. When CPU is idle, the process is maintained in a sleep state to conserve energy. TinyOS includes a tiny scheduler and a set of components. The scheduler schedules operation of those components. Each component consists of four parts: command handlers, event handlers, an encapsulated ﬁxed-size frame, and a group of tasks . Commands and tasks are executed in the context of the frame and operate on its state. Each component will declare its commands and events to enable modularity and easy interaction with other components. The current task scheduler in TinyOS is a simple FIFO mechanism whose scheduling data structure is very small, but it is power efﬁcient since it allows a processor to sleep when the task queue is empty and while the peripheral devices are still running. The frame is ﬁxed in size and is assigned statically.

It speciﬁes the memory requirements of a component at compile time and removes the overhead from dynamic assignment .Commands are nonblocking requests made to the low-level components. Therefore, commands do not have to wait a long time to be executed. A command provides feedback by returning status indicating whether it was successful (e.g., in the case of buffer overrun or of timeout). A command often stores request parameters into its frame and conditionally assigns a task for later execution. The occurrence of a hardware event will invoke event handlers. An event handler can store information in its frame, assign tasks, and issue high-level events or call low-level commands. Both commands and events can be used to perform a small and usually ﬁxed amount of work as well as to preempt tasks. Tasks are a major part of components. Like events, tasks can call low-level commands, issue high-level events, and assign other tasks. Through groups of tasks, TinyOS can realize arbitrary computation in an event-based model. The design of components makes it easy to connect various components in the form of function calls. This WNS operating system deﬁnes three type of components: hardware abstractions, synthetic hardware, and high-level software components. Hardware abstraction components are the lowest-level components. They are actually the mapping of physical hardware such as I/O devices, a radio transceiver, and sensors. Each component is mapped to a certain hardware abstraction. Synthetic hardware components are used to map the behavior of advanced hardware and often sit on the hardware abstraction components. TinyOS designs a hardware abstract component called the radio-frequency module (RFM) for the radio transceiver, and a synthetic hardware component called radio byte, which handles data into or out of the underlying RFM. An evaluation of TinyOS shows that it achieves the following performance gains or advantages:

- It requires very little code and a small amount of data.

- Events are propagated quickly and the rate of posting a task and switching the corresponding context is very high.

- It enjoys efﬁcient modularity.

2.Nes C (Programming language)

nesC (pronounced "NES-see") is a component-based, event-driven programming language used to build applications for the TinyOS platform. TinyOS is an operating environment designed to run on embedded devices used in distributed wireless sensor networks. nesC is built as an extension to the C programming language with components "wired" together to run applications on TinyOS. The name nesC is an abbreviation of "network embedded systems C".

The basic concepts behind nesC are:

1. Separation of construction and composition. Programs are built out of components, which are assembled ("wired") to form whole programs. Components have internal concurrency in the form of tasks. Threads of control may pass into a component through its interfaces. These threads are rooted either in a task or a hardware interrupt.

2. Specification of component behaviour in terms of set of interfaces. Interfaces may be provided or used by components. The provided interfaces are intended to represent the functionality that the component provides to its user, the used interfaces represent the functionality the component needs to perform its job.

3. Interfaces are bidirectional. They specify a set of functions to be implemented by the interface's provider (commands) and a set to be implemented by the interface's user (events). This allows a single interface to represent a complex interaction between components (e.g., registration of interest in some event, followed by a callback when that event happens). This is critical because all lengthy commands in TinyOS (e.g. send packet) are non-blocking; their completion is signaled through an event (send done). By specifying interfaces, a component cannot call the send command unless it provides an implementation of the sendDone event. Typically commands call downwards, i.e., from application components to those closer to the hardware, while events call upwards. Certain primitive events are bound to hardware interrupts.

4. Components are statically linked to each other via their interfaces. This increases runtime efficiency, encourages robust design, and allows for better static analysis of programs.

5.nesC is designed under the expectation that code will be generated by whole-program compilers. This should also allow for better code generation and analysis.

TinyOS is an event-driven operating system designed for sensor network nodes that have very limited resources (e.g., 8K bytes of program memory, 512 bytes of RAM).

## Introduction to Components

Every program in nesC is formed by  Components . There are two kinds of Components: Modules and Configurations. A Component is always a file written in nesC, that presents two sections, containing its specification and its implementation. However, the implementation section differs depending if we are talking about a Module or a Configuration:

Module implementation sections consist of nesC code that looks like C. Module code declares variables and functions, calls functions, and compiles to assembly code.

Configuration implementation sections consist of nesC wiring code, which connects components together. Configurations are the major difference between nesC and C (and other C derivatives).

All components have therefore two code blocks. The first block describes its signature, and the second block describes its implementation:

All components have therefore two code blocks. The first block describes its signature, and the second block describes its implementation:

module testP {

// signature

}

implementation {

// implementation

}

configuration testC {

// signature

}

implementation {

// implementation

}

Versions

1.0: Initial nesC release.

1.1:Concurrency support: async declarations, atomic statements, compile-time data-race detection, uniqueCount.

1.1.1: Better support for new platforms.

1.1.2: Maintenance release (minor features, bug fixes).

1.1.3: Network Types: platform-independent networking support.

Notation

The typewriter font is used for nesC code and for filenames. Single symbols in italics, with optional subscripts, are used to refer to nesC entities, e.g., “component K” or “value v”. The grammar of nesC is an extension the ANSI C grammar. We chose to base our presentation on the ANSI C grammar from Appendix A of Kernighan and Ritchie (K&R). We will not repeat productions from that grammar here. Words in italics are non-terminals and nonliteral terminals, typewriter words and symbols are literal terminals. The subscript opt indicates optional terminals or non-terminals. In some cases, we change some ANSI C grammar rules. We indicate this as follows: also indicates additional productions for existing non-terminals, replaced  by indicates replacement of an existing non-terminal. Explanations of nesC constructs are presented along with the corresponding grammar fragments. In these fragments, we sometimes use . . . to represent elided productions (irrelevant to the construct at hand). Appendix A presents the full nesC grammar. Several examples use the uint8 t and uint16 t types from the C99 standard inttypes.h file.

3.Sensor Simulator

Wireless Sensor Networks (WSNs), which consist of spatially distributed self-configurable sensors, perfectly meet the requirement. Since running real experiments is costly and time consuming, simulation is essential to study WSNs, being the common way to test new applications and protocols in the field. This survey illustrates some main-stream WSNs simulators including: NS-2, TOSSIM, EmStar, OMNeT++, J-Sim, ATEMU, and Avrora.

### Why use simulation in WSNs

Nowadays, the WSN is a hot research topic. Many network details in WSNs are not finalized and standardized. Building a WSNs testbed is very costly. Running real experiments on a testbed is costly and difficulty. Besides, repeatability is largely compromised since many factors affect the experimental results at the same time. It is hard to isolate a single aspect. Moreover, running real experiments are always time consuming. Therefore, WSNs simulation is important for WSNs development. Protocols, schemes, even new ideas can be evaluated in a very large scale. WSNs simulators allow users to isolate different factors by tuning configurable parameters.

Consequently, simulation is essential to study WSNs, being the common way to test new applications and protocols in the field. This leads to the recent boom of simulator development.

Basic Concepts

There are three types of simulation: Monte Carlo Simulation, Trace-Driven Simulation and Discrete-Event Simulation. The last two simulations are used commonly in WSN.

### Discrete-Event Simulations and Trace-Driven Simulation

Discrete-event simulation is widely used in WSNs, because it can easily simulate lots of jobs running on different sensor nodes. Discrete-event simulation includes some of components. This simulation can list pending events, which can be simulated by routines. The global variables, which describe the system state, can represent the simulation time, which allow the scheduler to predict this time in advance. This simulation includes input routines, output routines, initial routines, and trace routines. In addition, this simulation provides dynamic memory management, which can add new entities and drop old entities in the model. Debugger breakpoints are provided in discrete-event simulation, thus users can check the code step by step without disrupting the program operation.

Trace-Driven Simulation provides different services. This kind of simulation is commonly used in real system. The simulation results have more credibility. It provides more accurate workload; these detail information allow users to deeply study the simulation model. Usually, input values in this simulation constant unchanged. However, this simulation also contains some drawbacks. For example, the high-level detail information increases the complexity of the simulation; workloads may change, and thus the representativeness of the simulation needs to be suspicious. In this survey, seven main-stream simulation tools are categorize into this two types, the detail information are described in section

Simulator and Emulator

Simulator is universally used to develop and test protocols of WSNs, especially in the beginning stage of these designs. The cost of simulating thousands of nodes networks is very low, and the simulation can be finished within very short execution time. Both general and specialized simulators are available for uses to simulate WSNs. The tool, which is using firmware as well as hardware to perform the simulation, is called emulator. Emulation can combine both software and hardware implementation. Emulator implements in real nodes, thus it may provide more precision performance. Usually emulator has highly scalability, which can emulate numerous sensor nodes at the same time.

Simulation Tools

This section illustrates seven main-stream simulation tools used in WSNs: NS-2, TOSSIM, EmStar, OMNeT++, J-Sim, ATEMU, and Avrora, and analyzes the advantage and disadvantage of each simulation tool.

### NS-2

NS-2 is the abbreviation of Network simulator version two, which first been developed by 1989 using as the REAL network simulator. Now, NS-2 is supported by Defense Advanced Research Projects Agency and National Science Foundation. NS-2 is a discrete event network simulator built in Object-Oriented extension of Tool Command Language and C++. People can run NS-2 simulator on Linux Operating Systems. NS-2 is a popular non-specific network simulator can used in both wire and wireless area. This simulator is open source and provides online document.

## 2. OMNeT++

OMNeT++ is a discrete event network simulator built in C++. OMNeT++ provides both a noncommercial license, used at academic institutions or non-profit research organizations, and a commercial license, used at "for-profit" environments. This simulator supports module programming model. Users can run OMNeT++ simulator on Linux Operating Systems, Unix-like system and Windows. OMNeT++ is a popular non-specific network simulator, which can be used in both wire and wireless area. Most of frameworks and simulation models in OMNeT++ are open sources.

3. J-Sim

J-Sim is a discrete event network simulator built in Java. This simulator provides GUI library, which facilities users to model or compile the Mathematical Modeling Language, a “text-based language” written to J-Sim models. J-Sim provides open source models and online documents. This simulator is commonly used in physiology and biomedicine areas, but it also can be used in WSN simulation. In addition, J-Sim can simulate real-time processes.

## 4.  Avrora

Avrora is a simulator specifically designed for WSNs built in Java. Similar to ATEMU, Avrora can also simulate AVR-based microcontroller MICA2 sensor nodes. This simulator was developed by University of California, Los Angeles Compilers Group. Avrora provides a wide range of tools that can be used in simulating WSNs. This simulator combines the merits of TOSSIM and ATEMU, and limits their drawbacks. Avrora does not provide GUI. Avrora also supports energy consumption simulation. This simulator provides open sources and online documents. However, this simulator has some drawbacks. It does not have GUI. In addition, Avrora can not simulate network management algorithms because it does not provide network communication tools.